IE 448 Exam

Fall 1999

Due Wed, 11/24/99 (you can fax or mail it – postmark of 11/24 is sufficient)

This exam is a take home exam.  You may use your books and notes to solve the problems.  You may not talk to or communicate in any other way with anyone else about the exam.  If you have questions, you can email or call me.  Include this page on with your exam and signature below.

I have not received any help or clarification for this exam other than that which is authorized by the professor.

Your Signature

Date

- Do problem 5.39 in the text.  You can use any method to solve the problem (note the text says Huffman but you can use any).  Turn in a pneumatic circuit showing all switches and valves.  Also turn in a ladder logic diagram to control the circuit.

- Assume the following:

- all 3 cylinders use Momentary (pulse) 2 position valves with dual solenoids

- each cylinder has 2 limit switches to determine whether it is extended or retracted

- your goal is to minimize the activation time of the solenoids

- Do Problem 5.43 in the text.

- You can use any method to solve the problem.  Turn in a pneumatic circuit showing all switches and valves.  Also turn in a ladder logic diagram to control the circuit.

- Assume the following:

- Both cylinders use sustained output valves (2 position with spring return)

- Both cylinders return to the “minus” position when at rest

- each cylinder has 2 limit switches to determine whether it is extended or retracted

- Find the simplest sum of products equation that yields 1’s for the following terms:

- a’b’c’d’e’f ’	a’bc’d’e’f ’	ab’cd’e’f ’

- a’b’c’d’e’f		a’bc’d’e’f	ab’cd’ef ’

- a’b’c’d’ef		a’bc’d’ef	a’b’cd’ef

- a’bcd’ef

- the following terms can never happen and should be don’t cares:

- a’b’cd’e’f ’	a’b’cd’ef ’	a’b’cd’e’f

- a’bcd’e’f	ab’cd’e’f	ab’cd’ef

- Develop a pneumatic circuit and relay ladder logic circuit to perform the following sequence:

On initial power up you should retract A and Extend B

Extend A

Wait 5 seconds

Retract  B

Wait 10 seconds

Simultaneously Retract A and Extend B

Start the sequence over again.

Assume you are implementing this circuit with relays (not a PLC).  Assume Momentary output 2 position valves.  You have switches to detect when A and B are extended and retracted.   There is no Start switch.